Additional BlueDisk Notes

The 5.25” or 3.5” disk included with the BlueDisk controller is NOT bootable.  The test programs are located in the ‘TEST.PROGRAMS’ directory.

The CD-ROM contains the .PDF BlueDisk Manual and many other files you might find useful, such as ShrinkIt v3.4.  You can use ShrinkIt to help extract the disk images located on the CD-ROM when you have moved them to the Apple.  The BlueDisk and a 3.5” floppy drive are well suited for this!  We recommend the PC based Apple file utility CiderPress to assist you which is also located on the CD-ROM.  When you decide to purchase it tell Andy McFadden we sent you!

*** Important Note:  When turning off your computer we HIGHLY recommend ejecting the floppy disk from the floppy drive BEFORE powering off the computer.  We also recommend turning the floppy drive off just after the computer.  When turning on the computer we recommend turning on the floppy drive just before the computer.  If these two procedures are not observed then there is a possibility that the BlueDisk controller may accidentally write to the floppy disk as the computer is powering down.  Ejecting the disk BEFORE turning off the power will help prevent any data loss.  Resetting the power to the floppy drive each time the computer is powered on and off may also be required to insure your Apple will boot properly.

Quick Reference to the MicroDrive/BlueDisk switch settings

Thread the IDE cable through the one of your Apple’s access holes in the rear of the case.  Be careful not to damage the gray part of the cable end as it can be cut or torn quite easily by the Apple’s case if care isn’t taken when installing the cable.  You may also want to use paper, tape or something similar to help protect the gray part of the cable when passing it though the Apple’s case.  The black part of the cable is very tough.  It can be cut, but the soft gray cable inside of the back cable is jacketed.  A small cut in the black cable isn’t an issue.  A small cut in the gray cable can be.

Once the cable end is inside your computer, you now need to determine where Pin 1 is on your Controller.  On the BlueDisk Pin 1 in on the lower part of the card by the Slot Connector (aka – Edge Connector).

ReactiveMicro.com

Revised: 8-28-6