IIe Enhancement Kit

First open the Apple IIe so you can see the Motherboard.  Be sure the computer is off, but leave it plugged in.  There isn't any risk of electric shock, but it will reduce the possibility of static that could damage your kit or computer.

You'll need to locate the CPU, CD, EF and Video chips on your Motherboard in order to replace/upgrade them.

Let's start with the CPU.  It's the long 40 Pin chip with 6502 written on it.  Looking at your Motherboard it's just below Slot 3 and the Motherboard may also be marked with CPU written on it by the chip.  Make a note of the 'notch' or indent on one end of the chip.  This tells us where Pin 1 is.  If you put the chip in backward you will damage it!

First touch your Power Supply to discharge any static that may be on your person.  Make sure to do this before touching any chips or the Motherboard.  The more often you touch the Power Supply the better!  Using a small screwdriver or prying device remove your CPU and install the new one.  Be sure to pry on the chip and not the socket.  Sometimes it can be hard to tell if you have the screwdriver in the correct location.  Make sure the notch on the new CPU is oriented in the same direction.  You may want to lie something, like paper, on the Motherboard to prevent any damage when prying the CPU loose.

Now we need to locate the Video ROM.  It's the smallest of the chips in your kit.  You'll find it on your Motherboard just a few chips below your CPU.  Follow the line of chips from the CPU towards the keyboard.  You will see the Video ROM at the very end of the Motherboard.  The Motherboard may also be marked with VIDEO written on it by the chip.  Make a note of the 'notch' or indent on one end of the chip.  This tells us where Pin 1 is.  If you put the chip in backward you will damage it!

Using the same method as before, carefully remove the Video chip and install the new one.

Now we need to locate the CD and EF ROM's.  On the right hand side of the Motherboard you'll see two chips that are the same size as the two remaining chips in the kit.  The Motherboard should also be marked with CD and EF on it.

Using the same method as before, carefully remove the ROM chips and install the new ones.

Before you put your computer back together, double check that all the notches on all the new chips are pointing in the SAME direction as all the other chips on the Motherboard.  If you put one chip in backwards you will damage it and this is the last time you will be able to check for any mistakes before it's too late!

You're done!  Make sure to keep your original chips in a safe, static free place.

Please don’t hesitate to contact us on our forum for any support questions that you may have.

Thanks again for purchasing the IIe Enhancement Kit.

UltimateApple2.com & ReActiveMicro.com

Revised: 7-24-6