VGA Cable Pinouts

15 Pin HD VGA

19 Pin IDC Connector - Female                                       D-Sub Connector – Female

1 ---------------BLUE-------------- 3

2 -----------Monitor ID 2---------- 4

3 --------------GREEN------------- 2

4 ------GREEN GROUND------- 7

5 ----------------RED--------------- 1

6 ---------RED GROUND-------- 6

9-------------GROUND------------5

10-------------H-SYNC-------------13

11-----------Monitor ID 0----------11

12 ------------V-SYNC------------ 14

RGB Cable Pinouts

15 Pin RGB

19 Pin IDC Connector - Female                                       D-Sub Connector – Female

1 ------------------------------------- 1

2 ------------------------------------- 9

3 ------------------------------------- 2

4 ------------------------------------ 10

5 ------------------------------------- 3

6 ------------------------------------ 11

7 ------------------------------------- 4

8 ------------------------------------ 12

9 ------------------------------------- 5

10 ------------------------------------ 13

11 ------------------------------------- 6

12 ------------------------------------ 14

13 ------------------------------------- 7

14 ------------------------------------ 15

15 ------------------------------------- 8

Note: A 15 Pin RGB Crimp (IDC) connector should be used for the RGB Cable.  Just crimp the Ribbon Cable oriented to Pin 1 on each connector.