German Character ROM Differences: Apple IIe Video Unenhanced - German - 341-0161-A - 2764.bin

The difference consists of eight characters:

@ -> �

[ -> �

\ -> �

] -> �

{ -> �

| -> �

} -> �

~ -> �

Note: If you can�t see the above letters in German, make sure Word is set to �Western European (Windows)� for Text Encoding.

i.e. the German //e uses ISO 646 encoding in the German variant.

All the rest, including MoseText in advanced //e's, are unchanged

between the two halves of the chip.

I have to add a caveat: The German characters in that ROM are badly

designed and optically don't fit in well with the rest, especially the

sharp s letter "?" is really botched and sticks out like a sore thumb. I

assume they were done by somebody either not from Germany, or a

technician who didn't have an eye for typography at all.

Note: Fix ROM file is called: Apple IIe Video Enhanced - Improved German - 2764.bin

The CD and EF ROMs are not different between the German and US versions,

neither in the unenhanced nor in the enhanced //e.

> Now what about the Keyboard ROM?  I see the Keyboard ROM is also a different

> part number then the USA IIe's.  Will the Keyboard ROM need to be updated as

> well?

No. It is identical between the unenhanced and enhanced German //e.

Source: Linards Ticmanis (ticmanis@gmx.de)

http://compgroups.net/comp.sys.apple2/german-iie-question-and-rom-jedec-link/1124574

2006-7-15

Updated: 2014-4-18 HSC