External CF Drive

Installation:

To install the External CF Drive to an IDE Controller, such as the MicroDrive, is quite easy.

First, unpack the drive.

Next thread the IDE cable through the one of your Apple’s access holes in the rear of the case.  Be careful not to damage the gray part of the cable end as it can be cut or torn quite easily by the Apple’s case if care isn’t taken when installing the cable.  You may also want to use paper, tape or something similar to help protect the gray part of the cable when passing it though the Apple’s case.  The black part of the cable is very tough.  It can be cut, but the soft gray cable inside of the back cable is jacketed in a metal sheath.  A small cut in the black cable isn’t an issue.  A small cut in the gray cable can be.

Once the cable end is inside your computer, you now need to determine where Pin 1 is on your IDE Controller.  On the MicroDrive Pin 1 in on the lower part of the card by the Slot Connector (aka – Edge Connector).  Install the cable to the IDE Controller.

Now plug in the AC power cord to the wall and drive.

Finished!

Note:  When booting the Apple, make sure to insert a CF Card and turn on the External CF Drive FIRST, otherwise the CF card inserted in to the Drive may not be recognized by your IDE Controller.  You may also be required to turn the External CF Drive OFF and then ON when also turning your computer OFF and then ON.  Different Controllers respond differently to this situation and you may need to experiment to determine what your particular Controller requires.

Please don’t hesitate to contact us if we can be of any help setting up the External CF Drive, answer questions or resolving any technical issues.  Thanks again for order!

ReactiveMicro.com

Revised: 8-13-6