ROM0/1 Battery Pack

1 – Open the IIgs and remove the Power Supply.  There is a plastic clip on the front of the case that holds the Power Supply in place.  Releasing the clip releases the Power Supply.

2 – Locate the battery on the Motherboard.  It is where the Power Supply used to be.

3 – Using a wire cutter or snip to cut the axial leads as close to the battery as possible.  Leave the longest leads as possible on the Motherboard.  You can also choose to solder the battery pack to the Motherboard at this time.

4 – Using a small pliers, straighten the old battery leads on the Motherboard.  The straighter they are the easier it will be to install the AMP MTA connectors from the Battery Pack on them.

5 – Install 3 ‘AAA’ batteries in the Battery Pack.

6 –  *** Installing the Battery Pack backwards (incorrect polarity) will damage the system!***  Note the ‘+’ and ‘-‘ signs on the Motherboard.  The Positive (+) side of the leads are towards the back of the computer.  Install the RED lead from the Battery Pack on the Positive (+) lead on the Motherboard.  Install the remaining Black lead of the Battery Pack to the Negative (-) lead on the Motherboard.

7 – Install Power Supply and set Control Panel settings.  Finished.

Notes:  After turning the system off your setting should be saved.  If anything changes then a battery might be installed backwards in the Battery Pack.  If not the there is possibility that the RAM chip that holds the System Settings might be defective.  Using top quality batteries in the Battery Pack will provide years of service without needing to be changed.  Leaking batteries can pose an acid damage issue and should be changed every few years or at least inspected every few months.

Thank You For Your Order!

Revised: 2-24-6

www.GSE-Reactive.com